- Which of the following is not electromagnetic radiation?

- Radio Waves

- Light

- Infrared radiation

- X-rays

- Acoustic waves

- Optical fibers have minimum loss near 1.5 micrometers. What is the frequency that corresponds to that wavelength?

- 200 MHz

- 20 GHz

- 200 GHz

- 20 THz

- 200 THz

- An electron-volt is the energy needed to move an electron across a potential of 1 V. Suppose you could convert all the energy from moving an electron across a potential of 1.5 V into a photon. What would its wavelength be?

- 0.417 micrometers

- 0.5 micrometers

- 0.827 micrometers

- 1.21 micrometers

- 1.2399 micrometers

4. Light that passes from air into glass is

- Reflected

- Refracted

- Absorbed

- Scattered

5. Light is confined within the core of a

simple clad optical fiber by

- Refraction

- Total internal reflection at the outer edge of the cladding

- Total internal reflection at the core-cladding boundary

- Reflection from the fiber’s plastic coating

6. An optical fiber has a core with

refractive index of 1.52 and a cladding

with index of 1.45. Its number aperture

is

- 0.15

- 0.20

- 0.35

- 0.46

- 0.70

7. Zircon has a refractive index of 2.1.

What is the critical angle for total

internal reflection in air?

- 8 degrees

- 25 degrees

- 32 degrees

- 42 degrees

- 62 degrees

8. The output of a 20-km fiber with

attenuation of 0.5 dB/km is 0.005 mW.

What is the input power to the fiber?

- 0.5 mW

- 0.1 mW

- 0.05 mW

- 0.03 mW

- 0.01 mW

9. What fraction of the input power

remains after light travels through 100

km of fiber with 0.3 dB/km attenuation?

- 0.1%

- 0.5%

- 1%

- 5%

- 10%

10. If a 1 cm glass plate transmits 90% of

the light that enters it, how much light

will emerge from a 10 cm slab of the

same glass?

- 0%

- 9%

- 12%

- 35%

- 80%

11. What happens to light that is scattered

in an optical fiber?

- It escapes from the sides of the fiber

- Glass atoms absorb its energy

- Glass atoms store the light and release it later

- It’s reflected back towards the light source

- It excites acoustic waves in the glass

12. What effect does dispersion have?

- Scattering of light out the sides of fiber

- Stretching of signal pulses that increases with distance

- Shrinking of signal pulses that become shorter with distance

- Attenuation of signal pulses