- What is the half-acceptance angle for a large-core step-index fiber with core index of 1.5 and cladding index of 1.495?

- 4.7o

- 7.0o

- 9.4o

- 11o

- 14o

- Modal dispersion is largest in what type of fiber?

- Step-index multimode

- Graded-index multimode

- Step-index single-mode

- Dispersion-shifted single-mode

- polarization-maintaining

3.  A fiber has modal dispersion of 20 ns/km. If an instantaneous light pulse traveled

through 8 km of such fiber, what would the pulse length be at the end?

- 8 ns

- 20 ns

- 40 ns

- 80 ns

- 160 ns

4.  What is the maximum data rate that the 8 km length of fiber in #3 would carry?

- 160 Mbit/s

- 20 Mbit/s

- 16 Mbit/s

- 6 Mbit/s

- 4.4 Mbit/s

5. What guides light in multimode graded-index fibers?

- Total internal reflection

- Mode confinement in the cladding

- Refraction in the region where core index decreases to match the cladding index

- The optics that couple light into the fiber

6. What is the maximum allowable core diameter for a step-index single-mode fiber

operating at 1.3 micrometers, with core index of 1.5 and cladding index of air?

- 0.34 micrometers

- 0.89 micrometers

- 3.0 micrometers

- 4.8 micrometers

- 5.5 micrometers

7.  What is the maximum core diameter for a step-index single-mode fiber operating

at 1.3 micrometers, with core index of 1.5 and cladding index of 1.495?

- 0.89 micrometers

- 3.0 micrometers

- 4.1 micrometers

- 8.1 micrometers

- 10.1 micrometers

8. What is the cutoff wavelength of a single-mode step-index fiber with core diameter

of 8 micrometers, core index of 1.5 and cladding index of 1.495?

- 0.89 micrometers

- 1.15 micrometers

- 1.28 micrometers

- 1.31 micrometers

- 1.495 micrometers

9. What is the cutoff wavelength of a single-mode step-index fiber with core diameter

of 8 micrometers, core index of 1.5 and cladding index of 1.496?

- 0.89 micrometers

- 1.15 micrometers

- 1.28 micrometers

- 1.31 micrometers

- 1.495 micrometers