1 . What is the most essential property of all glass?

a. It is a noncrystalline solid. Page 128

b. It is a crystalline solid.

c. It must be transparent.

d. It must be made of pure silica.

e. It must have a refractive index of 1.5.

2. What type of fiber is drawn from a preform made by fusing a low-index tube

onto a higher-index rod?

a. step-index single-mode

b. graded-index multimode

c. dispersion-shifted

d. short-distance imaging and illumination Page 129

e. all of the above

3. What impurity levels are required in fused silica for communications fibers?

a. less than 0. 1%

b. less than 0.001%

c. one part per million page 130

d. ten parts per billion

e. one part per billion

4. What is done to make a depressed-clad fiber?

a. The fiber is flattened by rollers to depress it before the cladding is applied.

b. The refractive index in the core is depressed by adding germanium.

c. The refractive index in the inner part of the cladding is depressed by

adding fluorine. Page 131

d. The fiber is clad with a low-index plastic.

e. The entire fiber is made of pure silica because it has the lowest refractive

index of any glass.

5 . How are preforms for communications fibers made?

a. by the rod-in-tube method

b. by soot deposition in a fused silica tube

c. by soot deposition on the outside of a ceramic rod

d. by vapor axial deposition on the end of a rod

e. by methods b, c, and d pages 132 - 133

6. What is not used as a cladding for silica fiber?

a. silica with refractive index depressed by adding fluorine

b. silica with refractive index increased by adding germanium

c. hard plastic

d. soft plastic

e. pure silica cladding on a core doped to have higher refractive index page 135

7. What type of fiber could transmit the highest laser power?

a. step-index silica fiber with a 550-um core page 137

b. hard-clad silica fiber with a 100-um core

c. all-plastic fiber with a 1000-um core

d. single-mode fiber

e. plastic-clad silica fiber with a 200-um core

8. What is the lowest loss of laboratory all-plastic fibers?

a. 1 dB/km

b. 50 dB/km page 137

c. 150 dB/km

d. 500 dB/km

e. 1 dB/m

9. At what wavelength does PMMA plastic fiber have the lowest loss?

a. 500 nm page 139

b. 650 nm

c. 850 nm

d. 1.3 um

e. 1.55 um

10 . Why would you use fluorozirconate fibers?

a. because you couldn’t find any other fibers

b. because their attenuation is 0.001 dB/km

c. to transmit near-infrared wavelengths of 2 -5 um where silica fibers have

high loss

d. to transmit infrared wavelengths near 10 um

e. to compensate for losses in plastic fibers page 141

11 . What types of fibers can be used at wavelengths longer than 2 um?

a. fused silica, fluoride, and silver halide

b. plastic, fused silica, and fluoride pages 140-142

c. fluoride, chalcogenide, and silver halide

d. plastic, fused silica, and chalcogenide

e. only plastic-clad silica

12. What is the main advantage of graded-index plastic fiber over other plastic fibers?

a. higher bandwidth page 139

b. uses less plastic

c. more flexible

d. as clear as graded-index glass fiber

e. larger core diameters

13. What is a holey fiber?

a. a fiber made from a flawed preform that contains tiny air bubbles, which

scatter light from the sides

b. a fiber that guides light through holes in a plastic with a very low

refractive index

c. a photonic fiber in which the glass contains tiny holes that block light

transmission at certain wavelengths pages 142-144

d. a theoretical proposal, which has yet to be demonstrated

e. another name for a hollow optical waveguide used to transmit the 10.6-um

wavelength of carbon-dioxide lasers

14. In what way is a planar waveguide like an optical fiber?

a. It guides light through a region of high refractive index by total internal

Reflection. Page 144

b. It has attenuation below 0.5 dB/km at 1.3 to 1.6 um.

c. It is a flexible structure.

d. It has a plastic coating to protect it from scratches.